Section Cover Page

Section 02931 2004-07-09 Trees, Shrubs and Ground Covers

This Master Specification Section contains:

.1	This Cover Page

.2	Specification Section Text:

1.	General

1.1	Related Sections

1.2	Reference Standards

  - Submittals

  - Soil Testing

1.5	Inspection by Minister

1.6	Delivery, Storage and Handling

1.7	Warranty

1.8	Maintenance Period

1.9	Measurement and Payment

2.	Products

2.1	Plants - General

2.2	Plants - Characteristics

2.3	Plant Measurement

2.4	Plants - Soil Balls

2.5	Soil

2.6	Water

2.7	Planting Media

2.8	Plant Accessories

3.	Execution

3.1	Preparation

3.2	Preparation of Plant Beds and Tree Pits

3.3	Planting

3.4	Supporting Trees

3.5	Pruning

3.6	Maintenance

3.7	Acceptance

3.8	Clean-up

Applicable Standards

1.	CAN/CGSB-51.20-M87	Thermal Insulation, Polystyrene, Boards and Pipe Covering

These reference documents may be available for reference purposes in the Technical Services Branch Library, Alberta Infrastructure, 3rd Floor, 6950 - 113 Street, Edmonton; Telephone:  (780) 644-5748.

1.	General

1.1	RELATED SECTIONS

.1	Topsoil Placement and Grading:	Section 02911.

.2	Seeding:	Section 02921.

.3	Sodding:	Section 02922.

.4	Exterior Landscape Maintenance:	Section 02936.

1.2	REFERENCE STANDARDS

.1	Nomenclature:  to "International Code of Nomenclature for Cultivated Plants".

.2	Canadian Standards For Nursery Stock:  latest edition by Canadian Nursery Trades Association / Landscape Canada.

.3	Alberta Yards & Gardens, What to Grow published by Alberta Agriculture, Food and Rural Development, Agdex 200/32-1.

.4	Pruning in Alberta published by Alberta Agriculture, Food and Rural Development Agdex 270/24-1.

1.3	SUBMITTALS

.1	Comply with requirements of Division 1.

SPEC NOTE:  Include cross-reference to Division 1 where general requirements for Submittals are

specified in Division 1.

.2	Submit to Minister 500 g sample of [soil mix] [and] [topsoil] intended to be used on this project.

.3	Store soil sample in a clean container with label indicating source of material, date of sampling and project name.

.4 		Provide Minister with written documentation indicating original nursery and place of growth and sources of supply for all plants intended to be used when directed.

.5		Samples of planting accessories when directed by Minister, including product data.

1.4	SOIL TESTING

SPEC NOTE:  Select whether Minister  or Contractor will be responsible for testing of Contractor supplied topsoil or soil mix.

.1		Minister will appoint and pay for services of accredited testing laboratory to perform soil quality analysis on Contractor supplied [soil mix] [and] [topsoil].

[OR]

.1	Contractor will arrange and pay for services of accredited testing laboratory, approved by Minister, to perform complete soil quality analysis on imported [soil mix] [and] [topsoil].  Submit original copy of soil analysis to Minister.

.2	Soil analysis shall include results for: existing soil nutrients; soil salinity expressed as electrical conductivity; pH value; organic matter; soil texture; fertility recommendations; and recommended soil conditioning amendments.

.3	Where stockpiled topsoil exists on site and is used for planting media Minister will perform soil tests when necessary.

1. 5	INSPECTION BY MINISTER

.1	The Minister will inspect plants at the following stages:

.1	Plants at local source of supply or place of growth in Alberta.

.2	Proposed plant locations before excavating.

.3	Installed plants before commencement of maintenance period.

.4	At end of maintenance period.

.2	Notify Minister 48 hours in advance of each required inspection.  Contractor shall ensure required plants will be available or meet specifications before arranging inspection for selection and tagging of plants.

.3	Approval of plants at source of supply does not impair the right of Minister to inspect plants on site during project construction.  Any plant which has been damaged, does not conform to specifications or where Minister installed tags are missing will be rejected.

1.6	DELIVERY, STORAGE AND HANDLING

.1	Handle plants with reasonable care and skill to prevent injuries to trunk, branches, roots, rootballs and containers.  Trees with damaged, broken or abraded trunks or branches will be subject to rejection.

.2	Protect plants during shipment with tarpaulin, windscreen or other suitable covering.  Carefully tie in all branches before transporting.  Take all precautions to prevent excessive drying from sun and wind and breakage from wind and equipment.  Pad all points of contact between plant and equipment.

.3	Provide adequate site protection to rootballs, roots, trunks, branches and leaves of plants from drying, frost, equipment, breakage, vandalism and other damage.

.4	Install plants within 24 hours of delivery to site.  Store and heel plants in with topsoil or mulch in suitable location when planting cannot be completed on time.  Keep all plants moist until planted.  Plants not planted within required timeframe, weather permitting, may be subject to rejection.

.5	Do not install plants whose soil balls have been cracked or broken before or during planting or when burlap ropes required in connection with their transplanting have been removed.  Do not use plants damaged during contact with equipment, or plants that are wilted, windburned or stressed.

.6	Replace damaged or rejected plants at no cost to Minister.

.7	Do not transport trees in open trucks when the temperature is in excess of 25oC, or at speeds in excess of 60 km/h.

1.7	WARRANTY

.1	Replace plants when found dead or not in healthy and satisfactory growing condition during warranty period specified in General Conditions of Contract.

1.8	MAINTENANCE PERIOD

SPEC NOTE:  When maintenance requirements are for more than six weeks, delete this article and include Section 02936  - Exterior Landscape Maintenance.

.1	Maintain plants from time of planting until [six] [       ] weeks after date of Interim Acceptance of the Work.

1.9	MEASUREMENT AND PAYMENT

SPEC NOTE:  For Unit Price Contracts only.

.1	Quantity of plants supplied and installed on site will be paid by [lump sum] [each individual plant].  Unit bid price shall include plant, planting media, planting accessories, warranty and all other work, equipment and materials required to complete the Work.

.2	Quantity of soil mix supplied and installed on site for plant beds will be paid by [lump sum] [square metre].  Unit bid price shall include soil mix, excavation and disposal, warranty and all other work, equipment and materials required to complete the Work.

.3	Quantity of plant maintenance will be paid [per week] [       ].  Unit bid price shall include all work, equipment and materials required to complete maintenance requirements.

.4	[                            ]

2.	Products

2.1	PLANTS - GENERAL

.1	Supply nursery grown plants that are grown in Alberta or Prairie province, when available.

.2	Supply plants conforming to federal and provincial regulations.

.3	Nursery Grown Plants:

.1	All plants shall be No. 1 Grade, nursery grown, under proper cultural practices with respect to fertile soil, ample spacing, regular cultivation, weed, pest and disease control, adequate moisture and pruning in accordance with good horticultural practices as advocated by the Canadian Nursery Trades Association and the Landscape Alberta Nursery Trades Association.

.2	Plants true to genus, species and variety, structurally sound, well balanced, healthy, vigorous, of normal growth habits, densely foliated and healthy well developed root system.

.3	Plants free of disease, insect infestations, insect eggs, rodent damage, sun scald, frost cracks and other abrasions or scars to bark.

.4	Heeled-in plants or plants from cold storage will not be accepted.

.5	Plants which have been top worked, unnaturally sheared or colour treated are not acceptable.

.6	Plants from native stands, orchards, wood lots, neglected nurseries or which have not received proper cultural maintenance are not acceptable.

.4	Collected Plants:

.1	Native plants indigenous to area where they are to be transplanted.

.2	Trees from reasonably open stands with well developed crowns.

.3	Branched in upper 60% of overall tree height.

.4	Dig with soil balls 20% greater than specified for nursery grown stock.

2.2	PLANTS - CHARACTERISTICS

.1	Deciduous Trees:  straight bodied trees according to their natural habit of growth.  Double leaders not acceptable.  Trunks clean and free of stubs, decay, splits or other damage.

.2	Coniferous Trees:  trees having straight central leaders with dense and uniform branching that begins no higher than 200 mm from base of tree.  Double or broken leaders not acceptable.

.3	Clump Trees:  with three or more main stems originating from common base at ground line.

.4	Shrubs:  natural form, typical of genus, species and variety; minimum of four canes.

.5	Vines:  with at least four runners, each with minimum length of 30 cm.

.6	Ground Covers:  with healthy well developed tops, size proportionate to root requirements, typical of species and variety.

.7	Herbaceous Plants:  with healthy crowns, size proportionate to root requirements, typical of species and variety, not less than two years old and twice transplanted.

2.3	PLANT MEASUREMENT

.1	All plants shall conform to measurements specified in plant list.

.2	Measure trees and shrubs, in centimetres, with branches in their normal positions.

.3	Height and spread dimensions, in centimetres, specified refer to main body of plant.

.4	Caliper size, in millimetres, takes precedence over height.  Take caliper measurement 30 cm above ground.

.5	Measure [vines] [ground cover plants] [herbaceous plants] by:

.1	Units of plants of specified age

.2	Size of clump

.3	Number, and length of runners in centimetres.

.6	Supply container grown plant in minimum sized container when indicated on plant list.  Plants supplied in smaller than specified container will be rejected.

2.4	PLANTS - SOIL BALLS

.1	Container grown plants:

.1	Grown in containers for minimum of three months.

.2	Established root system which will "hold" soil when removed from container.

.3	Container sized in proportion to plant size.

.4	Root bound plants are not acceptable.

.2	Balled and burlapped plants:

.1	Soil balls tightly secured with burlap, heavy twine and rope, or burlap wire baskets and rope.  Plant trunks shall be in centre of the root ball.

.2	Protect balls against exposure to elements.

.3	Deliver with firm natural ball of soil and free from pernicious perennial plants and their roots.

SPEC NOTE:  Edit minimum ball sizes for deciduous and coniferous trees to match specified trees.

.3	Minimum ball sizes for nursery grown deciduous trees:

| Caliper (mm) | Min. Ball Diameter (cm) |
| --- | --- |
| 40 | 60 |
| 50 | 70 |
| 60 | 70 |
| 70 | 80 |
| 80 | 90 |
| 90 | 100 |
| 100 | 100 |
| 125 | 120 |
| 150 | 150 |
| 175 | 175 |
| 200 | 200 |

.4	Minimum ball sizes for nursery grown coniferous trees:

| Height (cm) | Min. Ball Diameter (cm) |
| --- | --- |
| 100 | 45 |
| 125 | 50 |
| 150 | 60 |
| 175 | 70 |
| 200 | 90 |
| 250 | 100 |
| 300 | 120 |
| 350 | 140 |
| 400 | 165 |

.5	Adjust ball size according to growing habits of plants.  Listed soil ball sizes are minimum and for use on nursery grown trees under proper nursery conditions.  Multi-stem trees and pine trees require soil ball minimum two sizes larger.

.6	Ball size shall be sufficiently large to contain at least 75% of fibrous root system with a ball depth not less than 50% of ball diameter.

.7	Trees 150 mm caliper and over shall be root pruned during previous two successive growing seasons prior to delivery.

2.5	SOIL

.1	Topsoil:  fertile, friable, shredded, screened loam soil meeting the following requirements:

.1	Not less than 6% organic material.

.2	pH value ranging from 5.9 to 7.0.

.3	Non-toxic to plant growth.

.4	E.C.-Salinity reading shall not exceed 1.5.

.5	Soil texture:  loam soil as defined by Canadian System of Soil Classification.

.6	Reasonably free from subsoil, slag, clay, stone, lumps, live plants, roots, sticks, quack-grass, noxious weeds, and foreign material.

.2	Stockpiled Topsoil:  topsoil stockpiled [on site] [off site] meeting specified requirements and approved by Minister.  [Screen topsoil prior to use on site].

SPEC NOTE:  Delete Item 2.5.2 if no suitable stockpile topsoil exists on or off site.

.3	Peat Moss:  decomposed plants, fairly elastic and homogeneous, free of decomposed colloidal residue, wood, sulphur and iron.  Minimum of 80% organic matter by mass, pH value between 4.5 and 6.0.

.4	Sand:  washed course sand, medium to course textured, free of impurities, chemical or organic matter.

.5	Bonemeal:  finely ground; chemical analysis:  4-20-0.

2.6	WATER

.1	Water:  clean water free of contaminants that may inhibit germination and tree growth.

.1	General purpose use: on site source supplied and paid for by Minister.

.2	Water for water hauling equipment: supplied and paid for by Contractor.

2.7	PLANTING MEDIA

.1	Soil mix:  3 parts fertile topsoil; 1 part horticultural peat moss; 1 part sharp sand;

bonemeal – apply at 0.6 kg/m3 soil mix or 50g/m2 soil mix and thoroughly mix into soil.

2.8	PLANT ACCESSORIES

SPEC NOTE:  Edit plant accessories to include only the accessories that are required for project.

.1	Support posts:  T-rail studded 40 mm x 40 mm metal stakes, minimum 2.1 m long.

.2	Guy wire:  #12 or #14 pliable galvanized steel wire.

.3	Guy wire anchors:  T-rail metal stakes, minimum 600 mm long.

.4	Bark protector:  two ply reinforced black rubber garden hose 12 mm diameter, 75 mm wide fabric webbing or rubber belting and as directed by Minister.

.5	Flagging:  fluorescent orange flagging tape.

.6	Tree trunk guard:  expandable plastic trunk guard, approximately 300 mm long.

.7	Plant protection:  materials to protect plants from rodent, animal and sun damages applied according to manufacturer’s instructions:

.1	Galvanized wire mesh and fastener.

.2	Chemical repellent treatment containing thiram (fungicide) mixture.

.3	Perforated spiral plastic strip.

.4	Burlap plant wrap and fastener.

.8	Gravel for planters:  [20] [   ] mm washed crushed stone.

.9	Planter insulation:  extruded, rigid polystyrene board CAN/CGSB-51.20-M87, “Type 4”.

.10	Filter cloth:  black horticultural nonwoven fabric with a minimum tensile strength of 0.530 kN, apparent opening size of 0.15  to 0.21 mm and as approved by Minister.

.11	Mulch material:

.1	Coniferous wood mulch:  shredded mulch from [cedar] [fir] [pine] bark, varying in size from 25 to 100 mm in length.  Mulch shall be free of non-organic materials, wood preservatives or diseased wood.

.2	Deciduous wood chip mulch:  mulch containing chipped wood and bark varying in size from 50 to 80 mm.  Mulch shall be free of non-organic material, wood preservatives or diseased wood.

.3	[                        ]

.12	Edging material:  heavy duty black polyethylene edger with heavy 25 mm top bead, bottom v-lip, 135 mm width, c/w anchor stakes, connectors and other installation accessories.

.13	[                        ]

3.	Execution

3.1	PREPARATION

.1	Locate and stake locations of electrical services, utility lines and other underground obstructions.

.2	If underground services interfere with planting plan, notify Minister, stop work in area and await further instructions.

.3	Stake out locations of trees, shrubs and planting beds.  Locations of all plants shall be reviewed and approved by Minister on site before planting.  Plants installed without review and approval may require transplanting or relocation as directed by Minister at no additional cost.

.4	Protect bench marks and hubs which form part of legal site survey.

3.2	PREPARATION OF PLANT BEDS AND TREE PITS

.1	Shrub Beds:  excavate plant beds minimum of [450] [       ] mm deep.  Excavate planters minimum of 600 mm deep.  Remove excavated material from site.

.2	Trees and Specimen Shrubs:  excavate plant pits large enough for soil ball and 300 mm around and beneath soil ball.  Remove excavated material from site.

.3	Do not excavate plant pits unless trees are scheduled for installation the same day.  Cover and/or clearly flag all excavated plant pits left unattended for any length of time that may present a hazard.

.4	Place [topsoil] [soil mix] in planting beds to required elevations.  Slope grade away from building.

.5	Place [150] [       ] mm of gravel over approved subgrade in planters and cover with filter cloth.

.6	Install filter cloth over prepared plant beds only where indicated on drawing.

.7	Install edging material according to manufacturer’s instructions or detail at edge of plant beds where indicated on drawings.

3.3	PLANTING

.1	Scarify sub-grade soil within tree pit to depth of 100 mm to eliminate glazed walls before placing soil.  Cover bottom of each tree pit excavation with minimum of 300 mm of topsoil [soil mix] and firmly tamp.

.2	Place plants at same depth they were originally grown.  Face for best appearance.

.3	Measure minimum depth of plant pit from downward side of slope when planting on incline.

.4	Container plants:  remove containers and keep soil ball intact while placing plant in pit.  Where roots have circled the container, gently loosen outside roots before planting.

.5	Balled and burlapped plants:

.1	Keep burlap and twine intact while placing plant in pit.

.2	Tamp topsoil [soil mix] around ball in layers of 150 mm eliminating air voids.

.3	When 75% of topsoil [soil mix] has been placed fill hole with water.

.4	After water has completely penetrated soil, cut and fold burlap from top third of root ball before placing final layer of topsoil [soil mix].

.5	Remove all ropes, wires and binding from top third of root ball.

.6	Complete backfilling, build 100 mm high soil lip around outer edge of plant pit.  Fill with water and allow it to be absorbed.  In sodded areas, install trees with slight depression to create required saucer.

.7	Fill watering saucer with wood mulch to 75 mm depth.  Keep mulch material away from trunks and stems.

.8	Place [coniferous] [deciduous] wood mulch to 75 to 100 mm depth over plant bed, [where indicated on drawing].  Keep mulch material away from trunks and stems.

.9	Install tree trunk guard around base of each deciduous tree only when indicated in planting detail on drawing.

SPEC NOTE:  Remove clause 3.3.7 and 3.3.8 if no mulch is specified and clause 3.3.9 if tree guards are not required.

.10	Do not install any plants, except ground covers, vines and herbaceous plants, no closer than 750 mm to edge of walkways, driveways or building foundations.  Consider proximity to buildings and walks when installing trees.

3.4	SUPPORTING TREES

.1	Support trees immediately after installation.

.2		Trees under 300 cm in height use two support posts.  Place one support on prevailing wind side of tree and one support directly opposite.

.3		Trees 300 cm in height and over use three support posts.  Space posts equally around tree.

.4		Anchor support post minimum 600 mm into undisturbed ground.  Ensure support post is secure and vertical.  Replace all damaged support posts.

.5		Locate support posts away from trunk and branches.  Height of support shall correspond with tree height.

.6		Provide each tree support with a bark protector and tie secured at approximately 60% of tree height.  Keep ties taut and tree plumb.

.7	Deciduous trees over 100 mm in caliper and coniferous trees 350 cm in height and over  with approval from Minister:

.1	Guy with three double guys and anchors spaced equally around plant.

.2	Provide guy looped around plant trunk, encased in hose or rubber belting.

.3	Attach guys at 60% of plant height at angle of 45 degrees.

.4	Top of anchor to be flush with finished grade.

.5	Keep guys taut and plant plumb.

.6	Attach flagging tape to guys as safety precaution.

.8	Minister reserves the right to adjust the need, method, and quantity of support posts required to adequately stake each tree.

3.5	PRUNING

.1	Perform pruning in accordance with proper practices and standards described in Pruning in Alberta.  Consult with Minister before pruning.

.2	Prune to provide natural branching structure and to encourage healthy natural growth.  Do not prune terminal leader of plant or leave protruding stubs when pruning.

.3	All improperly pruned plants or plants pruned by Contractor without Minister’s authorization will be subject to rejection.  Contractor will replace rejected plants or rectify improper pruning as determined and directed by Minister.

.4	Limit pruning to removal of dead or injured branches, stray branches, double leaders, watersprouts and suckers and to compensate for loss of roots as a result of transplanting.

.5	Do not strip lower branches, raise up crown of trees, top or shear any plant.  All such improper pruning will result in rejection and replacement by Contractor.

3.6	MAINTENANCE

SPEC NOTE:  When maintenance requirements are for more than six weeks, delete this article and include Section 02936 - Exterior Landscape Maintenance.

.1	Cultivate non-mulched plant beds and tree pits in upper 40 mm of soil biweekly to maintain a loose friable soil.  Do not damage plants roots when cultivating.

.2	Remove and dispose of debris, rubbish, animal waste, dead and unhealthy plants weekly.  Eliminate and remove perennial grass and weeds including their roots.

.3	Edge plant beds evenly to depth of 100 mm monthly or as required to maintain original line.

.4	Individually deep water all large trees weekly or as required to maintain healthy condition.  Water shrubs, ground covers, perennials and other plants weekly or more frequently to keep the ground moist and the plants healthy.  Apply sufficient water to saturate root zone when watering.

.5	Areas with no irrigation system:  supply labour, all hoses and attachments necessary to provide adequate watering.

.6	Provide clean water and water hauling vehicle with proper attachments to provide efficient and adequate watering of plants.

.7	Maintain plant accessories in proper condition, including tree supports , and soil lips. Repair disturbed mulch or replace to maintain original mulch depth.

.8	Install appropriate plant protection materials where required and maintain in proper repair.

.9	Straighten all plants that lean or sag.  Replant as necessary or when directed by Minister.

.10	Control disease and insects.  Where possible use physical methods to eliminate cause of insect infestation.  If chemical treatment is necessary apply in accordance with manufacturer's recommendations and government legislation.

.11	Promptly replace plants that die or become unhealthy during the maintenance and warranty periods.

.12	Maintain daily maintenance log throughout contract.  Submit copy of log data to Minister each week for verification.  Record all maintenance activities performed on site.

.13	Minister will extend maintenance period at no additional cost when Contractor fails to: maintain an accurate log; submit log when required; or when unsatisfactory and inadequate maintenance occurs.

3.7	ACCEPTANCE

.1	Plants will be accepted by Minister at end of maintenance period provided:

.1	Plants meet requirements of the specifications.

.2	Plants are alive and in healthy, acceptable growing condition.

.3	Plants are properly planted, staked and pruned.

.4	Plants are recently and thoroughly watered.

.5	Plant pits and beds are cultivated and free of weeds, grass and debris.

.6	Planting accessories are properly installed and in good condition.

.7	All maintenance requirements have been provided.

3.8	CLEAN-UP

.1	Broom clean pavement and sidewalks on daily basis or as directed by Minister.  Keep all manhole covers and catch basins free of soil and debris.

.2	Remove from site excess materials, debris, soils, branches and equipment.

.3	Repair any damage resulting from work of this Contract.

.4	Remove all tags and labels from accepted plants to prevent girdling.

.5	Leave site in neat and clean condition.

END OF SECTION